|  | AUTÓNOMA DE QUERÉTARO UNIVERSITY ELECTROMECHANICS ENGINEERING SERVOMECHANISMS 298 Trapezoidal aceleration profile generation |  |
| --- | --- | --- |

Lab Objectives

- Generate conventional trapezoidal acceleration profiles.

- Understand the effects of changing the design parameters in the shape of the generated motion dynamics.

Procedure

- Design a trapezoidal acceleration profile utilizing the motion dynamics tool. The parameters are:

| Parameter | Value |
| --- | --- |
| FsHz | 1000 |
| Xf(mm) | 300 |
| Vm(mms) | 25 |
| Am(mms2) | 19 |
| Jm(mms3) | 40 |

- Modify the parameters of acceleration and jerk and observe the changes in motion dynamics.

Required time: 1 hour

Equipment

- Motion dynamics tool

Quiz

- What happen in the profile if top acceleration parameter is increased?

- What happen in the profile if top acceleration parameter is decreased?

- What happen in the profile if top jerk parameter is increased?

- What happen in the profile if top jerk parameter is decreased?

- Which are the differences between trapezoidal speed profile and trapezoidal acceleration profile?