|  | AUTÓNOMA DE QUERÉTARO UNIVERSITY ELECTROMECHANICS ENGINEERING SERVOMECHANISMS 298 Effects of changing NURBS’ degree |  |
| --- | --- | --- |

Lab Objectives

- Understand the relation of motion dynamics with the NURBS degree.

- Comprehend how the continuity of the motion dynamics affects the surface finishing of the workpiece.

Procedure

- Generate a 2D NURBS path, utilizing no more than 10 control points and the following parameters:

| Parameter | Value |
| --- | --- |
| FsHz | 1000 |
| Xf(mm) | 10 |
| Vm(mms) | 25 |
| Dg | 2 |

- Find the motion dynamics.

- Increase Dg in one.

- Repeat step 2 and step 3 until Dg=5.

- Comment the differences between the obtained motion dynamics.

Required time: 1 hour

Equipment

- Motion dynamics tool

Quiz

- How affects Dg the shape of generated path?

- How is related the motion dynamics with the Dg?

- Which are the repercussions on the workpiece finishing of using NURBS-based movements with different degree?