|  | AUTÓNOMA DE QUERÉTARO UNIVERSITY ELECTROMECHANICS ENGINEERING SERVOMECHANISMS 298 Performance comparison of different profiles |  |
| --- | --- | --- |

Lab Objectives

- Understand the effects of motion dynamics on a simulated servomotor through utilizing several motion profiles in the simulation.

Procedure

- Generate three motion profiles. The same designed in LABPRACTICE_3_298, LABPRACTICE_4_298 and LABPRACTICE_5_298.

- Open the simulation module and set the following servomotor transfer function:

Gs=15.91 s2+376.60 s+32222s2−1.907 s+0.9072

- Select a sample frequency of Fs=1000Hz.

- Tune the PID controller and utilize the obtained parameters in the simulation (see LABPRACTICE_1_247).

- Select the following parameters for simulating dynamic torque:

| Parameter | Value |
| --- | --- |
| CFradmm | 6.2832 |
| J∗(Kg⋅m2) | 1x10−4 |
| B (N⋅mrads) | 2.5x10−4 |

- Simulate each motion profile.

- Compare the controller performance for each case and comment the results.

Required time: 3 hours

Equipment

- Motion dynamics tool

Quiz

- Which of the three motion profiles have the lower tracking error?

- Which of the three motion profiles have the lower dynamic torque?

- Comment about the obtained results