|  | AUTÓNOMA DE QUERÉTARO UNIVERSITY ELECTROMECHANICS ENGINEERING SERVOMECHANISMS 298 Quadratic jerk profile generation |  |
| --- | --- | --- |

Lab Objectives

- Generate quadratic jerk profiles and understand the effects of changing the design parameters in the shape of the generated motion dynamics.

- Understand the differences between quadratic jerk profile and the conventional motion profiles.

Procedure

- Design a quadratic jerk profile utilizing the motion dynamics tool. The parameters are:

| Parameter | Value |
| --- | --- |
| FsHz | 1000 |
| Xf(mm) | 300 |
| Vm(mms) | 25 |
| Am(mms2) | 19 |
| Jm(mms3) | 40 |

- Modify the parameters of acceleration and jerk and observe the changes in motion dynamics.

- Compare the obtained motion dynamics with the obtained in LABPRACTICE_3_298 and LABPRACTICE_4_298.

Required time: 1 hour

Equipment

- Motion dynamics tool

Quiz

- What happen in the profile if top acceleration parameter is increased?

- What happen in the profile if top acceleration parameter is decreased?

- What happen in the profile if top jerk parameter is increased?

- What happen in the profile if top jerk parameter is decreased?

- Which are the differences between quadratic jerk profile and conventional profiles?